一、线缆类型：MIC1 to MIC8

所有线缆都需要进行样件认证，认证合格后再生产正式产品。

表1：线缆种类

|  | 信号分类与电学特性 |  | MI线缆规格 |  |  |  | 线缆耐压 测试 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 用途 | 额定电压(V) | 额定电流值(A) | MIC# | 线缆外径（mm） | 端接头针芯数量 | 端头数量 | 测试电压(V) |
| 仪器（热电偶） | 50 | 0.5 | MIC1 | 1.5 | 2 | 2 | 200 |
| 仪器设备 |  |  | MIC1-hj | 1.5 | 2 | 1 | 200 |
| 仪器设备 | 50 | 0.5 | MIC2 | 1.6 | 2 | 2 | 200 |
| 控制/电源线 | 300 | 20 | MIC3 | 3 | 1 | 2 | 1000 |
|  |  |  | MIC4 | 6 | 2 | 2 | 1000 |
| 控制/电源线 | 600 | 4 | MIC5 | 3 | 2 | 2 | 1200 |
|  |  |  | MIC6 | 4 | 4 | 2 | 1200 |
| 控制/电源线 | 1000 | 2 | MIC7 | 3 | 1 | 2 | 2000 |
|  |  |  | MIC8 | 6.4 | 4 | 2 | 2000 |

二、 ITER LEVI 矿物质绝缘（MI）线缆首批采购范围

表2：ITER LEVI 矿物质绝缘（MI）线缆首批采购范围

| 序号 | 产品描述 | 产品型号 |  | 线缆长度 | 数量（根） | 线缆外径（mm） | 端接头针芯直径 (mm) | 端接头针芯数量 | 端头数量 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | LEVI MI线缆 | MIC1 | 015A | 2.710m±1cm | 10 | 1.5 | / | 2 | 2 |
| 2 |  | MIC1-hj | 015B | 2.735m ±1cm | 2 | 1.5 | / | 2 | 1 |
| 3 |  | MIC1-hj | 015B | 0.473m ±1cm | 4 | 1.5 | / | 2 | 1 |
| 4 |  | MIC1-hj | 015B | 0.261 ±1cm | 12 | 1.5 | / | 2 | 1 |
| 5 |  | MIC1-hj | 015B | 0.836 ±1cm | 21 | 1.5 | / | 2 | 1 |
| 6 |  | MIC2 | 016A | 0.6m ±1cm | 9 | 1.6 | / | 2 | 2 |
| 7 |  | MIC7 | 030B | 0.434m ±1cm | 4 | 3 | 2.1 | 1 | 2 |
| 8 |  | MIC5 | 030C | 0.9m ±1cm | 4 | 3 | 1.2 | 2 | 2 |
| 9 |  | MIC5 | 030C | 1.915m ±1cm | 6 | 3 | 1.2 | 2 | 2 |
| 10 |  | MIC6 | 040A | 1.902m±1cm | 46 | 4 | / | 4 | 2 |

备注：1.以上数量和尺寸是最终交付的做好接头之后的最终合格的交付数量和尺寸，总价包括生产过程中间的线缆和端头损耗。2.每种规格的线缆需额外生产1-2件用于测试的样件。3.每种规格的线缆最终交付长度可能与表2有所差异，以甲方最终需求为准。4.所有正式交付的线缆产品均以直线状态交付。

三、技术要求

3.1线缆总成要求

[RQ‑1] 线缆总成长度与标称长度公差：±10 mm。

[RQ‑2] 工厂验收试验 (FAT) 必须测量每根线缆总成长度，测量精度优于 ±2 mm。

[RQ‑3] 所有线缆总成的电气指标可参考表3，以甲方最终定稿版为准。

表3：线缆电气指标

| A | B | C | D | E | F | G | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MIC# | Rated Voltage | Rated Current | Withstand Voltage for Cables (Dielectric strength) | Insulation Resistance (Room ambient environmental conditions) | Short-Circuit Current value (A) – Values from IEEE 317 Standard | Short-Circuit Current value(A) – if applicable | Thermal Test (A2.t) |
| MIC#1 | 50 V | 0.5 A | 200V RMS test voltage @ 50Hz | > 100 G Ω @ 300 VDC | Not applicable | Not applicable | Not applicable |
| MIC#1-hj | 50 V | 0.5 A | 200V RMS test voltage @ 50Hz | > 100 G Ω @ 300 VDC | Not applicable | Not applicable | Not applicable |
| MIC#2 | 50 V | 0.5 A | 200V RMS test voltage @ 50Hz | > 100 G Ω @ 300 VDC | Not applicable | Not applicable | Not applicable |
| MIC#3 | 300V | 20 A | 1000V RMS @ 50Hz | > 100 G Ω @ 300 VDC | 858 A | 400 A | 11544 A2.t |
| MIC#4 | 300V | 20 A | 1000V RMS @ 50Hz | > 100 G Ω @ 300 VDC | 945 A | 400 A | 14032 A2.t |
| MIC#5 | 600V | 4 A | 1200V RMS @ 50Hz | > 100 G Ω @ 600 VDC | 111 A | 80 A | 190 A2.t |
| MIC#6 | 600V | 4 A | 1200V RMS @ 50Hz | > 100 G Ω @ 600 VDC | 217 A | 80 A | 721 A2.t |
| MIC#7 | 1000V | 2 A | 2000V RMS @ 50Hz | > 100 G Ω @ 1000 VDC | 485 A | 40 A | 3653 A2.t |
| MIC#8 | 1000V | 20A | 2000V RMS @ 50Hz | > 100 G Ω @ 1000 VDC | 552 A | 40 A | 4728 A2.t |

[RQ‑4] 线缆完成端接后应可直接进行弯曲作业。